Appendix E.	FORTRAN Language Program To Read an Archive Record

program testRead(arcfile,outfile)

! The program takes two arguments, specifying the input

! filename and the output filename (including paths.)

! The output file is written in the current working directory

! unless a path is included as part of the filename.

integer*2 j

integer*4 rec,eof,hr,min,sec

character*80 arcfile,outfile

! Common block /rec/ variables:

integer*4 cSum,ihd(6,4)

integer*2 cSumFlag,major,status(10),qual(16),minor(16),mdf(40,16)

real*4 analog(17),head(27,4),ssLoc(2,16),mep0(9,16),mep90(9,16),

+    mepOmni(4,16),ted0(8,16),ted30(8,16),ted0s(8,4),ted30s(8,4),

+    tedback(2,4),tedfx(7,16)

common /rec/cSum,ihd,cSumFlag,major,status,qual,minor,

+    mdf,analog,head,ssLoc,mep0,mep90,mepOmni,ted0,ted30,

+    ted0s,ted30s,tedback,tedfx

eof = 0

open(unit=7,access='direct',recl=2544,file = arcfile,status='old')

open(unit=11,access='sequential',file = outfile,

+     status='unknown',form='formatted')

do rec = 1,3

call readArcSub(7,rec,eof)

if(eof.eq.1) goto 100

do j=1,4

hr = ihd(4,j)/3600000

min = (ihd(4,j) - hr*3600000)/60000

sec = (ihd(4,j) - hr*3600000 - min*60000)/1000

write(11,200),ihd(2,j),ihd(3,j),hr,min,sec,

+                  head(2,j),head(3,j),ihd(6,j)

enddo

write(11,300)"mep0:"

do j=1,16

write(11,220)j,mep0(1,j),mep0(2,j),mep0(3,j),mep0(4,j),

+          mep0(5,j),mep0(6,j),mep0(7,j),mep0(8,j),mep0(9,j)

enddo

write(11,300)"mep90:"

do j=1,16

write(11,220)j,mep90(1,j),mep90(2,j),mep90(3,j),

+          mep90(4,j),mep90(5,j),mep90(6,j),mep90(7,j),mep90(8,j),

+          mep90(9,j)

enddo

write(11,300)"mepOmni:"

do j=1,16

write(11,220)j,mepOmni(1,j),mepOmni(2,j),mepOmni(3,j),

+       mepOmni(4,j)

enddo

write(11,300)"ted0:"

do j=1,16

write(11,220)j,ted0(1,j),ted0(2,j),ted0(3,j),ted0(4,j),

+          ted0(5,j),ted0(6,j),ted0(7,j),ted0(8,j)

enddo

write(11,300)"ted30:"

do j=1,16

write(11,220)j,ted30(1,j),ted30(2,j),ted30(3,j),

+          ted30(4,j),ted30(5,j),ted30(6,j),ted30(7,j),

+          ted30(8,j)

enddo

write(11,300)"ted0s:"

do j=1,4

write(11,220)j,ted0s(1,j),ted0s(2,j),ted0s(3,j),

+          ted0s(4,j),ted0s(5,j),ted0s(6,j),ted0s(7,j),ted0s(8,j)

enddo

write(11,300)"ted30s:"

do j=1,4

write(11,220)j,ted30s(1,j),ted30s(2,j),ted30s(3,j),

+          ted30s(4,j),ted30s(5,j),ted30s(6,j),ted30s(7,j),

+          ted30s(8,j)

enddo

write(11,300)"tedback:"

do j=1,4

write(11,220)j,tedback(1,j),tedback(2,j)

enddo

write(11,300)"tedfx:"

do j=1,16

write(11,220)j,tedfx(1,j),tedfx(2,j),tedfx(3,j),

+           tedfx(4,j),tedfx(5,j),tedfx(6,j),tedfx(7,j)

enddo

write(11,300)"----------------------------------------------"

enddo

200 format(i4,1x,i3,3(1x,i2),1x,f8.3,1x,f8.3,1x,i7)

220 format(i4,9(1x,f8.2))

300 format(A)

100 close(7)

close(11)

end

subroutine readArcSub (unit,rnum,eof)

! -----------------------------------------------------------

!   readArcSub.f returns a NOAA POES SEM2 data record from an

!               already opened file in packed binary format.

!

!   Calling from C:  It is recommended that C users use the

!                    C routine: unpackSem2.c instead.

!

!   Compilation: f77 -c readArcSub.f

!

!

!   Programmer:  Sue Greer

!

! -----------------------------------------------------------

byte arcBytes(2544)

integer*4 unit,rnum,eof,ios

integer*4 arc(2544),i,j,jj,k,m,mf,mo,mp0,mp90,qf,tf

integer*4 b1,b2,b3,b4

integer*2 onebyte

real*4 cf ! Conversion factor (.0001)

real*4 cnvrt(256)

! Common block /rec/ variables:

integer*4 cSum,ihd(6,4)

integer*2 cSumFlag,major,status(10),qual(16),minor(16),mdf(40,16)

real*4 analog(17),head(27,4),ssLoc(2,16),mep0(9,16),mep90(9,16),

+    mepOmni(4,16),ted0(8,16),ted30(8,16),ted0s(8,4),ted30s(8,4),

+    tedback(2,4),tedfx(7,16)

common /rec/cSum,ihd,cSumFlag,major,status,qual,minor,

+    mdf,analog,head,ssLoc,mep0,mep90,mepOmni,ted0,ted30,

+    ted0s,ted30s,tedback,tedfx

cf = 0.0001

b1=256*256*256

b2=256*256

b3=256

b4=1

data cnvrt /0.0,1.0,2.0,3.0,4.0,5.0,6.0,7.0,8.0,9.0,10.0,11.0,

+12.0,13.0,14.0,15.0,16.0,17.0,18.0,19.0,20.0,21.0,22.0,23.0,24.0,

+25.0,26.0,27.0,28.0,29.0,30.0,31.0,32.0,34.5,36.5,38.5,40.5,42.5,

+44.5,46.5,48.5,50.5,53.0,56.0,59.0,62.0,65.5,69.5,73.5,77.5,81.5,

+85.5,89.5,93.5,97.5,101.5,106.5,112.5,118.5,124.5,131.5,139.5,

+147.5,155.5,163.5,171.5,179.5,187.5,195.5,203.5,213.5,225.5,

+237.5,249.5,263.5,279.5,295.5,311.5,327.5,343.5,359.5,375.5,

+391.5,407.5,427.5,451.5,475.5,499.5,527.5,559.5,591.5,623.5,

+655.5,687.5,719.5,751.5,783.5,815.5,855.5,903.5,951.5,999.5,

+1055.5,1119.5,1183.5,1247.5,1311.5,1375.5,1439.5,1503.5,1567.5,

+1631.5,1711.5,1807.5,1903.5,1999.5,2111.5,2239.5,2367.5,2495.5,

+2623.5,2751.5,2879.5,3007.5,3135.5,3263.5,3423.5,3615.5,3807.5,

+3999.5,4223.5,4479.5,4735.5,4991.5,5247.5,5503.5,5759.5,6015.5,

+6271.5,6527.5,6847.5,7231.5,7615.5,7999.5,8447.5,8959.5,9471.5,

+9983.5,10495.5,11007.5,11519.5,12031.5,12543.5,13055.5,13695.5,

+14463.5,15231.5,15999.5,16895.5,17919.5,18943.5,19967.5,20991.5,

+22015.5,23039.5,24063.5,25087.5,26111.5,27391.5,28927.5,30463.5,

+31999.5,33791.5,35839.5,37887.5,39935.5,41983.5,44031.5,46079.5,

+48127.5,50175.5,52223.5,54783.5,57855.5,60927.5,63999.5,67583.5,

+71679.5,75775.5,79871.5,83967.5,88063.5,92159.5,96255.5,100351.5,

+104447.5,109567.5,115711.5,121855.5,127999.5,135167.5,143359.5,

+151551.5,159743.5,167935.5,176127.5,184319.5,192511.5,200703.5,

+208895.5,219135.5,231423.5,243711.5,255999.5,270335.5,286719.5,

+303103.5,319487.5,335871.5,352255.5,368639.5,385023.5,401407.5,

+417791.5,438271.5,462847.5,487423.5,511999.5,540671.5,573439.5,

+606207.5,638975.5,671743.5,704511.5,737279.5,770047.5,802815.5,

+835583.5,876543.5,925695.5,974847.5,1023999.5,1081343.5,

+1146879.5,1212415.5,1277951.5,1343487.5,1409023.5,1474559.5,

+1540095.5,1605631.5,1671167.5,1753087.5,1851391.5,1949695.5,

+1998848.0/

eof = 0

if (rnum.lt.1) goto 100

10 read(unit,rec=rnum,iostat=ios) arcBytes

if(ios.ne.0) then

eof = 1

goto 100

endif

do i=1,2544

! Byte type goes from -128 to +127

! Convert to 8-bit integer

onebyte = arcBytes(i)

arc(i) = onebyte

if(onebyte.lt.0) arc(i) = onebyte+256

enddo

* ------------------------------------------------------------------

*     ! Parse the data in arc()

* ------------------------------------------------------------------

! Checksum flag

cSumFlag = arc(1)*b1 + arc(2)*b2 + arc(3)*b3 + arc(4)

! Checksum

cSum = arc(5)*b1 + arc(6)*b2 + arc(7)*b3 + arc(8)

! Major frame number

major = arc(9)*b3 + arc(10)*b4

!print *,'cSumFlag, cSum, MF ',cSumFlag,cSum,major

! Status: instrument on/off, ifc, etc.

do i = 1,10

status(i) = arc(i+10)

!print *,i,status(i)

enddo

! Analog data, etc. 4-byte integers converted to reals

do i = 1,68,4

k = (i+1)/4 + 1

analog(k) = (arc(i+20)*b1+arc(i+21)*b2+arc(i+22)*b3+

+         arc(i+23))*cf

!print *,k,analog(k)

enddo

! Sub-satellite location, 4-byte integers converted to reals

k = 1

ssLoc(1,1) = (arc(k+88)*b1+arc(k+89)*b2+arc(k+90)*b3+

+         arc(k+91))*cf

ssLoc(1,2) = (arc(k+92)*b1+arc(k+93)*b2+arc(k+94)*b3+

+         arc(k+95))*cf

ssLoc(1,3) = (arc(k+96)*b1+arc(k+97)*b2+arc(k+98)*b3+

+         arc(k+99))*cf

ssLoc(1,4) = (arc(k+100)*b1+arc(k+101)*b2+arc(k+102)*b3+

+         arc(k+103))*cf

ssLoc(2,1) =(arc(k+104)*b1+arc(k+105)*b2+arc(k+106)*b3+

+         arc(k+107))*cf

ssLoc(2,2) =(arc(k+108)*b1+arc(k+109)*b2+arc(k+110)*b3+

+         arc(k+111))*cf

ssLoc(2,3) =(arc(k+112)*b1+arc(k+113)*b2+arc(k+114)*b3+

+         arc(k+115))*cf

ssLoc(2,4) =(arc(k+116)*b1+arc(k+117)*b2+arc(k+118)*b3+

+         arc(k+119))*cf

ssLoc(1,5) = (arc(k+120)*b1+arc(k+121)*b2+arc(k+122)*b3+

+         arc(k+123))*cf

ssLoc(1,6) = (arc(k+124)*b1+arc(k+125)*b2+arc(k+126)*b3+

+         arc(k+127))*cf

ssLoc(1,7) = (arc(k+128)*b1+arc(k+129)*b2+arc(k+130)*b3+

+         arc(k+131))*cf

ssLoc(1,8) = (arc(k+132)*b1+arc(k+133)*b2+arc(k+134)*b3+

+         arc(k+135))*cf

ssLoc(2,5) =(arc(k+136)*b1+arc(k+137)*b2+arc(k+138)*b3+

+         arc(k+139))*cf

ssLoc(2,6) =(arc(k+140)*b1+arc(k+141)*b2+arc(k+142)*b3+

+         arc(k+143))*cf

ssLoc(2,7) =(arc(k+144)*b1+arc(k+145)*b2+arc(k+146)*b3+

+         arc(k+147))*cf

ssLoc(2,8) =(arc(k+148)*b1+arc(k+149)*b2+arc(k+150)*b3+

+         arc(k+151))*cf

ssLoc(1,9) = (arc(k+152)*b1+arc(k+153)*b2+arc(k+154)*b3+

+         arc(k+155))*cf

ssLoc(1,10) = (arc(k+156)*b1+arc(k+157)*b2+arc(k+158)*b3+

+         arc(k+159))*cf

ssLoc(1,11) = (arc(k+160)*b1+arc(k+161)*b2+arc(k+162)*b3+

+         arc(k+163))*cf

ssLoc(1,12) = (arc(k+164)*b1+arc(k+165)*b2+arc(k+166)*b3+

+         arc(k+167))*cf

ssLoc(2,9) =(arc(k+168)*b1+arc(k+169)*b2+arc(k+170)*b3+

+         arc(k+171))*cf

ssLoc(2,10) =(arc(k+172)*b1+arc(k+173)*b2+arc(k+174)*b3+

+         arc(k+175))*cf

ssLoc(2,11) =(arc(k+176)*b1+arc(k+177)*b2+arc(k+178)*b3+

+         arc(k+179))*cf

ssLoc(2,12) =(arc(k+180)*b1+arc(k+181)*b2+arc(k+182)*b3+

+         arc(k+183))*cf

ssLoc(1,13) = (arc(k+184)*b1+arc(k+185)*b2+arc(k+186)*b3+

+         arc(k+187))*cf

ssLoc(1,14) = (arc(k+188)*b1+arc(k+189)*b2+arc(k+190)*b3+

+         arc(k+191))*cf

ssLoc(1,15) = (arc(k+192)*b1+arc(k+193)*b2+arc(k+194)*b3+

+         arc(k+195))*cf

ssLoc(1,16) = (arc(k+196)*b1+arc(k+197)*b2+arc(k+198)*b3+

+         arc(k+199))*cf

ssLoc(2,13) =(arc(k+200)*b1+arc(k+201)*b2+arc(k+202)*b3+

+         arc(k+203))*cf

ssLoc(2,14) =(arc(k+204)*b1+arc(k+205)*b2+arc(k+206)*b3+

+         arc(k+207))*cf

ssLoc(2,15) =(arc(k+208)*b1+arc(k+209)*b2+arc(k+210)*b3+

+         arc(k+211))*cf

ssLoc(2,16) =(arc(k+212)*b1+arc(k+213)*b2+arc(k+214)*b3+

+         arc(k+215))*cf

* -----------------------------------------------------

*      ! Header information

* -----------------------------------------------------

! Subsatellite latitude and longitude

head(2,1) = ssLoc(1,1)

head(2,2) = ssLoc(1,5)

head(2,3) = ssLoc(1,9)

head(2,4) = ssLoc(1,13)

head(3,1) = ssLoc(2,1)

head(3,2) = ssLoc(2,5)

head(3,3) = ssLoc(2,9)

head(3,4) = ssLoc(2,13)

do j = 1,4

m = 3

i = 1640+(j-1)*96

do k = 4,96,4

m = m+1

head(m,j) = (arc(i+1+k-4)*b1 + arc(i+2+k-4)*b2 +

+            arc(i+3+k-4)*b3 + arc(i+4+k-4))*cf

enddo

enddo

j = 1    ! Index for 8-second records

! ihd info, 4-byte integers  (1..5,1)

do i=1,20,4

k = (i+1)/4 +1

ihd(k,j) = (arc(i+216)*b1+arc(i+217)*b2+arc(i+218)*b3+

+         arc(i+219))

enddo

! Inclination (head(1,1))

head(1,j) =  (arc(237)*b1+arc(238)*b2+arc(239)*b3+

+         arc(240))*cf/10.

! Orbit number (6,1)

ihd(6,j) = (arc(241)*b1+arc(242)*b2+arc(243)*b3+

+         arc(244))

! Quality, 2-byte integer (1..4)

qf = 1

do i=1,7,2

qual(qf) = arc(i+244)*b3+arc(i+245)

qf = qf + 1

enddo

! Minor frame numbers (1..4)

mf = 1

do i = 1,7,2

minor(mf) = arc(i+252)*b3+arc(i+253)

mf = mf+1

enddo

! Missing data flags (1..40,1..4)

do i = 1,4

do n = 1,40

mdf(n,i) = arc(n-1+40*(i-1)+261)

enddo

enddo

j = 2

! ihd info, 4-byte integers (1..5,2)

do i=1,20,4

k = (i+1)/4 +1

ihd(k,j) = (arc(i+420)*b1+arc(i+421)*b2+arc(i+422)*b3+

+         arc(i+423))

enddo

! Inclination (head(1,2))

head(1,j) =  (arc(441)*b1+arc(442)*b2+arc(443)*b3+

+         arc(444))*cf/10.

! Orbit number  (ihd(6,2))

ihd(6,j) = (arc(445)*b1+arc(446)*b2+arc(447)*b3+

+         arc(448))

! Quality, 2-byte integer (5..8)

do i=1,7,2

qual(qf) = arc(i+448)*b3+arc(i+449)

qf = qf + 1

enddo

! Minor frame numbers (5..8)

do i = 1,7,2

minor(mf) = arc(i+456)*b3+arc(i+457)

mf = mf+1

enddo

! Missing data flags (1..40,5..8)

do i = 5,8

do n = 1,40

mdf(n,i) = arc(n-1+40*(i-5)+465)

enddo

enddo

j = 3

! ihd info, 4-byte integers (1..5,3)

do i=1,20,4

k = (i+1)/4 +1

ihd(k,j) = (arc(i+624)*b1+arc(i+625)*b2+arc(i+626)*b3+

+         arc(i+627))

enddo

! Inclination (head(1,3))

head(1,j) =  (arc(645)*b1+arc(646)*b2+arc(647)*b3+

+         arc(648))*cf/10.

! Orbit number  (ihd(6,3))

ihd(6,j) = (arc(649)*b1+arc(650)*b2+arc(651)*b3+

+         arc(652))

! Quality, 2-byte integer (9..12)

do i=1,7,2

qual(qf) = arc(i+652)*b3+arc(i+653)

qf = qf + 1

enddo

! Minor frame numbers (9..12)

do i = 1,7,2

minor(mf) = arc(i+660)*b3+arc(i+661)

mf = mf+1

enddo

! Missing data flags (1..40,9..12)

do i = 9,12

do n = 1,40

mdf(n,i) = arc(n-1+40*(i-9)+669)

enddo

enddo

j = 4

! ihd info, 4-byte integers (1..5,4)

do i=1,20,4

k = (i+1)/4 +1

ihd(k,j) = (arc(i+828)*b1+arc(i+829)*b2+arc(i+830)*b3+

+         arc(i+831))

enddo

! Inclination (head(1,4))

head(1,j) =  (arc(849)*b1+arc(850)*b2+arc(851)*b3+

+         arc(852))*cf/10.

! Orbit number (ihd(6,4))

ihd(6,j) = (arc(853)*b1+arc(854)*b2+arc(855)*b3+

+         arc(856))

! Quality, 2-byte integer (13..16)

do i=1,7,2

qual(qf) = arc(i+856)*b3+arc(i+857)

qf = qf + 1

enddo

! Minor frame numbers (13..16)

do i = 1,7,2

minor(mf) = arc(i+864)*b3+arc(i+865)

mf = mf+1

enddo

! Missing data flags (1..40,13..16)

do i = 13,16

do n = 1,40

mdf(n,i) = arc(n-1+40*(i-13)+873)

enddo

enddo

* -----------------------------------------------------

*      ! MEPED sensor data

* -----------------------------------------------------

j = 1

mp0 = 1

! Meped 0-deg telescope (1..9,1..4)

do i = 1,4

jj = (j-1)*4+i

do k = 1,9

if(mdf(k+1,jj).eq.1) then

mep0(k,jj) = -999.

else

mep0(k,jj) = cnvrt(1+arc(mp0+1032))

endif

mp0 = mp0+1

enddo

enddo

! Meped 90-deg telescope (1..9,1..4)

mp90 = 1

do i = 1,4

jj = (j-1)*4+i

do k = 1,9

if(mdf(k+10,jj).eq.1) then

mep90(k,jj) = -999.

else

mep90(k,jj) = cnvrt(1+arc(mp90+1068))

endif

mp90 = mp90+1

enddo

enddo

! Meped omnidirectional (1..4,1..4)

mo = 1

do i = 1,4

jj = (j-1)*4+i

do k = 1,4

mepOmni(k,jj) = cnvrt(1+arc(mo+1104))

mo = mo+1

enddo

if(mdf(20,jj).eq.1) mepOmni(1,jj) = -999.

if(mdf(21,jj).eq.1) mepOmni(2,jj) = -999.

if(mod(jj,2).eq.1.AND.mdf(22,jj).eq.1) mepOmni(3,jj) = -999.

if(mod(jj,2).eq.0.AND.mdf(22,jj).eq.1) mepOmni(4,jj) = -999.

enddo

j = 2

mp0 = 1

! Meped 0-deg telescope (1..9,5..8)

do i = 1,4

jj = (j-1)*4+i

do k = 1,9

if(mdf(k+1,jj).eq.1) then

mep0(k,jj) = -999.

else

mep0(k,jj) = cnvrt(1+arc(mp0+1184))

endif

mp0 = mp0+1

enddo

enddo

! Meped 90-deg telescope (1..9,5..8)

mp90 = 1

do i = 1,4

jj = (j-1)*4+i

do k = 1,9

if(mdf(k+10,jj).eq.1) then

mep90(k,jj) = -999.

else

mep90(k,jj) = cnvrt(1+arc(mp90+1220))

endif

mp90 = mp90+1

enddo

enddo

! Meped omnidirectional (1..4,5..8)

mo = 1

do i = 1,4

jj = (j-1)*4+i

do k = 1,4

mepOmni(k,jj) = cnvrt(1+arc(mo+1256))

mo = mo+1

enddo

if(mdf(20,jj).eq.1) mepOmni(1,jj) = -999.

if(mdf(21,jj).eq.1) mepOmni(2,jj) = -999.

if(mod(jj,2).eq.1.AND.mdf(22,jj).eq.1) mepOmni(3,jj) = -999.

if(mod(jj,2).eq.0.AND.mdf(22,jj).eq.1) mepOmni(4,jj) = -999.

enddo

j = 3

mp0 = 1

! Meped 0-deg telescope (1..9,9..12)

do i = 1,4

jj = (j-1)*4+i

do k = 1,9

if(mdf(k+1,jj).eq.1) then

mep0(k,jj) = -999.

else

mep0(k,jj) = cnvrt(1+arc(mp0+1336))

endif

mp0 = mp0+1

enddo

enddo

! Meped 90-deg telescope (1..9,9..12)

mp90 = 1

do i = 1,4

jj = (j-1)*4+i

do k = 1,9

if(mdf(k+10,jj).eq.1) then

mep90(k,jj) = -999.

else

mep90(k,jj) = cnvrt(1+arc(mp90+1372))

endif

mp90 = mp90+1

enddo

enddo

! Meped omnidirectional (1..4,9..12)

mo = 1

do i = 1,4

jj = (j-1)*4+i

do k = 1,4

mepOmni(k,jj) = cnvrt(1+arc(mo+1408))

mo = mo+1

enddo

if(mdf(20,jj).eq.1) mepOmni(1,jj) = -999.

if(mdf(21,jj).eq.1) mepOmni(2,jj) = -999.

if(mod(jj,2).eq.1.AND.mdf(22,jj).eq.1) mepOmni(3,jj) = -999.

if(mod(jj,2).eq.0.AND.mdf(22,jj).eq.1) mepOmni(4,jj) = -999.

enddo

j = 4

mp0 = 1

! Meped 0-deg telescope (1..9,13..16)

do i = 1,4

jj = (j-1)*4+i

do k = 1,9

if(mdf(k+1,jj).eq.1) then

mep0(k,jj) = -999.

else

mep0(k,jj) = cnvrt(1+arc(mp0+1488))

endif

mp0 = mp0+1

enddo

enddo

! Meped 90-deg telescope (1..9,13..16)

mp90 = 1

do i = 1,4

jj = (j-1)*4+i

do k = 1,9

if(mdf(k+10,jj).eq.1) then

mep90(k,jj) = -999.

else

mep90(k,jj) = cnvrt(1+arc(mp90+1524))

endif

mp90 = mp90+1

enddo

enddo

! Meped omnidirectional (1..4,13..16)

mo = 1

do i = 1,4

jj = (j-1)*4+i

do k = 1,4

mepOmni(k,jj) = cnvrt(1+arc(mo+1560))

mo = mo+1

enddo

if(mdf(20,jj).eq.1) mepOmni(1,jj) = -999.

if(mdf(21,jj).eq.1) mepOmni(2,jj) = -999.

if(mod(jj,2).eq.1.AND.mdf(22,jj).eq.1) mepOmni(3,jj) = -999.

if(mod(jj,2).eq.0.AND.mdf(22,jj).eq.1) mepOmni(4,jj) = -999.

enddo

* -----------------------------------------------------

*       TED uncalibrated energy flux, max channel

*       response, and max channel

* -----------------------------------------------------

! TED 0-deg and 30-deg uncalibrated energy flux

do k=1,4

do i = 1,4

j = (k-1)*4 + i  !(1..16)

ted0(1,j) = cnvrt(1+arc(1121+(i-1)*4+(k-1)*152))

if(mdf(27,j).eq.1) ted0(1,j) = -999.

ted0(2,j) = cnvrt(1+arc(1122+(i-1)*4+(k-1)*152))

if(mdf(29,j).eq.1) ted0(2,j) = -999.

ted0(3,j) = cnvrt(1+arc(1123+(i-1)*4+(k-1)*152))

if(mdf(31,j).eq.1) ted0(3,j) = -999.

ted0(4,j) = cnvrt(1+arc(1124+(i-1)*4+(k-1)*152))

if(mdf(33,j).eq.1) ted0(4,j) = -999.

ted0(5,j) = 1+arc(1137+(i-1)*4+(k-1)*152)

ted0(6,j) = 1+arc(1138+(i-1)*4+(k-1)*152)

if(mdf(35,j).eq.1) then

ted0(5,j) = -999.

ted0(6,j) = -999.

endif

ted0(7,j) = cnvrt(1+arc(1139+(i-1)*4+(k-1)*152))

if(mdf(36,j).eq.1) ted0(7,j) = -999.

ted0(8,j) = cnvrt(1+arc(1140+(i-1)*4+(k-1)*152))

if(mdf(37,j).eq.1) ted0(8,j) = -999.

ted30(1,j) = cnvrt(1+arc(1153+(i-1)*4+(k-1)*152))

if(mdf(28,j).eq.1) ted30(1,j) = -999.

ted30(2,j) = cnvrt(1+arc(1154+(i-1)*4+(k-1)*152))

if(mdf(30,j).eq.1) ted30(2,j) = -999.

ted30(3,j) = cnvrt(1+arc(1155+(i-1)*4+(k-1)*152))

if(mdf(32,j).eq.1) ted30(3,j) = -999.

ted30(4,j) = cnvrt(1+arc(1156+(i-1)*4+(k-1)*152))

if(mdf(34,j).eq.1) ted30(4,j) = -999.

ted30(5,j) = 1+arc(1169+(i-1)*4+(k-1)*152)

ted30(6,j) = 1+arc(1170+(i-1)*4+(k-1)*152)

if(mdf(38,j).eq.1) then

ted30(5,j) = -999.

ted30(6,j) = -999.

endif

ted30(7,j) = cnvrt(1+arc(1171+(i-1)*4+(k-1)*152))

if(mdf(39,j).eq.1) ted30(7,j) = -999.

ted30(8,j) = cnvrt(1+arc(1172+(i-1)*4+(k-1)*152))

if(mdf(40,j).eq.1) ted30(8,j) = -999.

enddo

enddo

* -----------------------------------------------------

*      ! Ted spectra and background

* -----------------------------------------------------

do j = 1,4

do i=1,8

ted0s(i,j) = cnvrt(1+arc(i+(j-1)*8+2024))

ted30s(i,j) = cnvrt(1+arc(i+(j-1)*8+2060))

enddo

tedback(1,j) = cnvrt(1+arc(j+2056))

tedback(2,j) = cnvrt(1+arc(j+2092))

enddo

do j=1,16

if(j.eq.1.OR.j.eq.5.OR.j.eq.9.OR.j.eq.13) then

do i=1,4

if(mdf(22+i,j).eq.1) then

do k=1,4

ted0s(i,k) = -999.

enddo

endif

enddo

endif

if(j.eq.3.OR.j.eq.7.OR.j.eq.11) then

do i=5,8

if(mdf(18+i,j).eq.1)then

do k=1,4

ted0s(i,k) = -999.

enddo

endif

enddo

endif

if(j.eq.2.OR.j.eq.6.OR.j.eq.10.OR.j.eq.14) then

do i=1,4

if(mdf(22+i,j).eq.1)then

do k=1,4

ted30s(i,k) = -999.

enddo

endif

enddo

endif

if(j.eq.4.OR.j.eq.8.OR.j.eq.12) then

do i=5,8

if(mdf(18+i,j).eq.1)then

do k=1,4

ted30s(i,k) = -999.

enddo

endif

enddo

endif

if(j.eq.15) then

if(mdf(1,j).eq.1) tedback(1,1) = -999.

if(mdf(23,j).eq.1) tedback(1,2) = -999.

if(mdf(26,j).eq.1) tedback(1,3) = -999.

if(mdf(25,j).eq.1) tedback(1,4) = -999.

if(mdf(24,j).eq.1) tedback(2,2) = -999.

endif

if(j.eq.16) then

if(mdf(1,j).eq.1) tedback(2,1) = -999.

if(mdf(26,j).eq.1) tedback(2,3) = -999.

if(mdf(24,j).eq.1) tedback(2,4) = -999.

endif

enddo

* -----------------------------------------------------

*      ! Ted flux

* -----------------------------------------------------

tf = 1

do i=1,4

do n = 1,7

do j = 1,4

k = (i-1)*4 + j

tedfx(n,k) = (arc(tf+2096)*b1 +

+                          arc(tf+2097)*b2 +

+                          arc(tf+2098)*b3 +

+                          arc(tf+2099))*cf

tf = tf + 4

!print *,n,k,tf+2096

enddo

enddo

enddo

* -----------------------------------------------------

*      ! Check missing data flags

* -----------------------------------------------------

100 continue

return

end