- Horizontal and Vertical Control Report

- Pribilof Shell Hash Survey

Bering Sea wavescape from the back deck of the Mt. Mitchell

Registry Number: N/A

Vessels: R/V Mt. Mitchell

Survey: Pribilof Shell Hash Survey

State: Alaska

General Locality: Bering Sea

Sublocality: Pribilof Islands

Survey Dates: June 16, 2009 to June 17, 2009

Lead Hydrographer: Chris Popham

- Vertical Control

The time meridian for this project was 000° longitude.  All measurements were made in Universal Time Coordinated (UTC).  No measurements were recorded using local time.  The local time meridian for the project was 135° West longitude and local time Alaska was offset from UTC by eight hours (Alaska = UTC - 8 hours).  The Vertical Control for this survey is based on the Mean Lower-Low Water (MLLW) datum established at the Village Cove tidal station on St.  Paul Island, AK (station ID #9464212).

- Tidal Corrections & Zoning

Sounding data were adjusted for tidal influence under provisions in the project instructions.  Verified tide data was downloaded from the Tides and Currents data retrieval website (http://tidesandcurrents.noaa.gov).  See Appendix I of this document for station information.

Secondary and tertiary tide stations were not established for this survey per instructions in the statement of work.  No tidal zoning was used.

There were no unusual tidal water levels or current conditions observed during the survey.

Tidal elevations were applied directly from the tide gauge mentioned above; no time, height, or zoning corrections were employed per the Statement of Work.

- Horizontal Control

The horizontal control datum used for this survey was the North American Datum of 1983 (NAD 83).  The projection used was Universal Transverse Mercator (UTM) Zone 2 North.

Sounding position control was determined using a Global Positioning System (GPS).  The primary source of navigation correctors was a C-NAV2050R unit, using the C-NAV global Correction Service Network.  The decimeter-level positioning accuracy meets survey requirements according to section 3.2.1 of NOAA Hydrographic Survey Specifications and Deliverables (HSSD) April 2008.  DGPS positioning was not available for this survey as Pribilof Canyon is located outside the range of all Coast Guard Continually Operating Reference Stations (CORS).

C-NAV system confidence checks were performed when the vessel was within range of the U.S.  Coast Guard Continually Operating Reference Station (CORS) located in Cold Bay, AK (Station ID BAY5).  This station operates at a frequency of 289 kHz and was received by an MBX-4 Differential Beacon Receiver.  See Appendix II of this document for the data sheet for this station.  The DGPS enabled POS M/V and the C-NAV positions were compared to a common node at regular intervals.  The differences in the Northing and Easting values were calculated and graphed.  A circle placed on the graph delineated the 95 percent confidence level; the positions did not exceed 5 meters + 5 percent of the depth of the given line in section 3.1 of the HSSD.  The DGPS confidence checks are provided in the Descriptive Report, Separates I: Acquisition Logs and Confidence Checks

Horizontal and Vertical Control Report

Pribilof Shell Hash Survey

All information contained in this Horizontal and Vertical Report for the Pribilof Shell Hash Survey has been reviewed and approved by me and is hereby respectfully submitted.

_____________________________________________

# Chris Popham, Lead Hydrographer

- TerraSond Ltd.

- Date        01/28/2010                      .