DESCRIPTIVE REPORT

Pribilof Shell Hash Survey

Bering Sea wavescape from the back deck of the Mt. Mitchell

Contract Number: DG133C08CQ0005

Vessels: R/V Mt. Mitchell

Survey: Pribilof Shell Hash Survey

State: Alaska

General Locality: Bering Sea

Sublocality: Pribilof Islands

Survey Dates: June 16, 2009 to June 17, 2009

Lead Hydrographer: Chris Popham

TABLE OF CONTENTS

A.	Area Surveyed	3

B.	Data Acquisition and Processing	4

B.1.	Equipment	4

B.2.	Quality Control	5

B.2.1.	Multibeam Bathymetry	5

B.2.2.	Crosslines	5

B.2.3.	Contemporary Survey Junctions	6

B.3.	Corrections to Echo Soundings	6

B.4.	Data Processing	6

C.	Vertical and Horizontal Control	7

D.	Results and Recommendations	7

D.1.	Chart Comparison	7

D.1.1.	New Features	8

D.1.2.	Charted Features	8

D.1.3.	Soundings	8

D.1.4.	Trends and Changeable Areas	9

D.1.5.	AWOIS Items Summary	11

D.1.6.	Features Labeled PA, ED, PD, or Rep.	11

D.2.	Additional Results	11

D.2.1.	Aids to Navigation	11

D.2.2.	Drilling Structures	11

D.2.3.	Comparison with Prior Surveys	12

D.2.4.	Bottom Samples	12

D.2.5.	Bridges and Overhead Cables	12

D.2.6.	Submarine Cables and Pipelines	12

LETTER OF APPROVAL	13

APPENDICES

I	Danger to Navigation Reports

II	Survey Feature Report

III	Progress Sketch

IV	Tide and Water Levels

V	Supplemental Survey Records and Correspondence

SEPARATES

I	Acquisition and Processing Logs

II	Sound Velocity Profile Data

III	Hydrographic Survey Letter Instructions/Statement of Work

IV	Crossline Comparisons

V	Side Scan Contact Listings and Images of Significant Contact

- Area Surveyed

A navigable area survey was conducted in two areas each to the northeast of St. George and St. Paul Islands, Alaska. This 48 hour survey served to field test the capacity of multibeam backscatter to delineate king crab habitat around the Pribilof Islands. This survey additionally served to deliver accurate hydrographic survey data with which to update the nautical charts of the assigned area.

The basic project area captured the transit to and from the Pribilof Canyon, two targeted areas on the north-east sides of both St. George and St. Paul islands, and the transit in between islands. The final project area covered approximately 50 square nautical miles between the St. George and St. Paul survey areas.

Figure 1 – Survey area for the Pribilof Shell Hash Survey. Background chart is NOAA ENC US3AK8CM 9th Edition.

Survey coverage was maximized for the short amount of time budgeted, and the large area to be investigated.  The areas surveyed were predicted to have high variability in bottom characteristics and provide the desired test of the bottom classification capabilities of the sonar.  Patches were surveyed until it was felt that a coherent area had been covered, at which point the vessel would transit to the next targeted area.  Full bottom coverage, consisting of 100% multibeam data was achieved within the limits of each targeted area for this survey.  One hundred percent backscatter data was acquired and stored by TerraSond, Ltd.  This survey has a maximum depth of 160 meters and a minimum depth of 30 meters below the Mean Lower Low Water (MLLW) tidal datum.  The R/V Mt. Mitchell collected 412 lineal nautical miles of mainscheme multibeam lines and 21 lineal nautical miles of crosslines between June 16, 2009 and June 17, 2009 for a total of 51.6 square nautical miles of coverage.  No bottom samples were collected in the project area.

For complete survey limits, refer to Figure 1 on the preceding page.

- Data Acquisition and Processing

  - Equipment

Bathymetry for this survey was acquired using the hydrographic survey vessel R/V Mt. Mitchell.

R/V Mt. Mitchell

The R/V Mt. Mitchell is a steel hull vessel, 70 meters length overall with a 12.7 meter beam and a 3.9 meter draft.  The ship was powered by two 1200 HP EMD/567C General Motors Diesel engines connected to Bird-Johnson controllable-pitch propellers operating between 10% and 80% pitch.  Electrical power was provided by two Detroit Diesel 300 kW generating plants located in the engine room and one Detroit Diesel 75 kW auxiliary generator.  Major systems used on the R/V Mt. Mitchell are listed in Table 1.

| VESSEL R/V Mt. Mitchell LOA: 70m, BEAM 12.7m, DRAFT: 3.9m |  |
| --- | --- |
| Equipment | Manufacturer & Model |
| Multibeam sonar | Kongsberg EM 710 |
| Positioning | Applanix POS MV V4 |
| Sound speed | AML Smart SV&T, SV Plus v2, Lockheed Martin Sippican XBT T-5 & XCTD-2 |
| Vessel attitude | Applanix POS MV - IMU 200 |

Table 1 - Major systems used aboard the R/V Mt. Mitchell.

Equipment performance details are provided in the Data Acquisition and Processing Report (DAPR), Sections B. Equipment and C. Quality Control.

  - Quality Control

    - Multibeam Bathymetry

No conditions with the potential for adversely affecting data integrity were encountered with the multibeam equipment used during this survey.

Multibeam confidence checks were conducted on the R/V Mt. Mitchell prior to beginning the survey and again at the end of survey operations to verify proper operation of the multibeam equipment.  Depth of the survey area, weather, and sea state were prohibitive of more frequent checks.  The confidence checks were performed by comparing nadir beam depths with lead line depths.  Additionally, a processed line surveyed by the R/V Mt. Mitchell was compared with a line run by the M/V Bluefin, another TerraSond, Ltd. survey vessel working on a NOAA charting contract (OPR-P188-TE-09) in Unimak Pass, AK at the end of the survey.  The results of these comparisons and the line acquisition logs detailing aspects of quality control for each survey line are contained in Separates I:  Acquisition and Processing Logs of this report.

Uncertainty surfaces were built in CARIS Hips and Sips, the uncertainty child layer was analyzed to verify the quality of the data in the surface.  The majority of the data was found to meet IHO Order 1 specifications.

Sound velocity (SV) profiles were taken for the full water column, and to the extent possible given the limited quantity of SV probes, were geographically and temporally distributed within the survey area to meet the criteria specified in NOS Hydrographic Surveys Specifications and Deliverables (HSSD) April 2008.  Sound speed profiles extended to 100% of the anticipated water depth.  No obvious data quality issues related to speed of sound measurements were encountered during the survey, however in post processing a clear SV anomaly was encountered. This anomaly is fully discussed in the Data Acquisition and Processing Report (DAPR)

A detailed discussion of multibeam system calibrations, speed of sound profiling, patch tests, data acquisition and processing is also provided in the DAPR.

    - Crosslines

133 Mainscheme lines totaling 412 lineal nautical miles and 9 lines totaling 21 lineal nautical miles of crosslines were run during the 2009 survey.  The ratio of the lineal nautical miles of crosslines to the lineal nautical miles of mainscheme lines, at 5.07%, just exceeds the 5% required by HSSD, Section 5.1.4.

Crossline analysis was conducted by creating a base surface of the mainscheme lines and a separate base surface using the crosslines.  The surfaces were then compared and the difference between the surfaces was computed using CARIS HIPS QC Report.  The surfaces were found to agree well at over 95% of nodes.  A comprehensive explanation of the crossline analysis process is in the DAPR.  The reports generated from the crossline analysis are in Separates IV: Crossline Comparisons of this report.

    - Contemporary Survey Junctions

This survey has no junctions with other surveys; therefore no junction comparison was performed.

  - Corrections to Echo Soundings

Any change to the corrections to echo soundings is described in detail in the DAPR.

Sounding data were reduced per project instructions using final tides from the National Water Level Observation Network (NWLON) tide station at Village Cove, Saint Paul, AK (946-4212).  Refer to the Horizontal and Vertical Control Report (HVCR) for tidal methods and operations.

  - Data Processing

The final depth information for this survey was submitted as a collection of CARIS CUBE surfaces which best represented the seafloor at the time of the 2009 survey.  All possible measures were taken to ensure data flow integrity and that the data was correctly processed.

Because of the nature of the survey, and the varied terrain of the area, line planning was not possible.  Swath width and spacing was adjusted by the operator to provide 100% bottom coverage in targeted areas.

Grids of varying resolution were created due to the depth ranges with in the surveyed area crossing NOS specified depth boundaries.  Grid spacing of 2, 4, and 8 meters were used for the BASE surfaces.

| Depth Range (m) | BASE Surface Resolution | Surface Name |
| --- | --- | --- |
| 27-52 | 2m | St_Paul_2m_CUBE_Order1 |
| 46-115 | 4m | St_Paul_4m_CUBE_Order1 |
| 27-52 | 2m | St_George_2m_CUBE_Order1 |
| 46-115 | 4m | St_George_4m_CUBE_Order1 |
| 27-52 | 2m | North_Transit_2m_CUBE_Order1 |
| 46-115 | 4m | North_Transit_4m_CUBE_Order1 |
| 27-52 | 2m | Southern_Transit_2m_CUBE_Order1 |
| 46-115 | 4m | Southern_Transit_2m_CUBE_Order1 |
| 103-350 | 8m | Southern_Transit_8m_CUBE_Order1 |

Table 2 –List of CUBE surfaces for Pribilof Shell Hash Survey.

Base Surfaces were named with the following naming convention: Fieldsheet  Name_Xm_CUBE_Order1_Final.  In the absence of sheet numbers, the survey area was divided into four fieldsheets (Southern Transit, St George, North Transit and St Paul).

The DAPR Sections B.2 Data Collection; and C: Quality Control contain a detailed discussion of the steps followed when acquiring and processing the 2009 survey data.

- Vertical and Horizontal Control

Sounding data were tide adjusted using verified data National Water Level Observation Network (NWLON) tide station at Village Cove, Saint Paul, AK (946-4212).  No tidal zoning methodology was applied.  Tidal considerations are described in detail in the project wide HVCR.

The horizontal control datum used for this survey is the North American Datum of 1983 (NAD 83).  The projection used was UTM, Zone 2 North.

Sounding position control was determined using a Global Positioning System (GPS).  The primary source of navigation correctors was a C-NAV 250R (which was receiving a globally corrected GPS (GcGPS) signal) as the survey area is outside of the published range of Coast Guard correction stations.  C-NAV corrected positions were compared to the GPS positions from United States Coast Guard differential GPS (DGPS) station at Cold Bay, AK broadcasting at 289 kHz when in range of the station.  A summary of DGPS confidence checks is provided in Separates I: Acquisition and Processing Logs included with this report.

- Results and Recommendations

  - Chart Comparison

The chart comparison was performed by comparing all RNC and ENC charts that intercept the project area to the surveyed data.  These charts are listed in Table 3

Discrepancies are discussed in context of the largest scale chart available and assumed to apply to the smaller scale charts unless specifically mentioned.

| ENC | RNC | Scale | Edition # | Issue Date | Last Update |
| --- | --- | --- | --- | --- | --- |
| US5AK8AM | 16381 | 1:50000 | 9 | 04/01/2006 | 11/14/2009 |
| US5AK8BM | 16382 | 1:50000 | 11 | 04/01/2006 | 11/14/2009 |
| US3AK8CM | 16380 | 1:200,000 | 15 | 11/01/2009 | 11/14/2009 |
| US2AK5FM | 16011 | 1:1,023,188 | 37 | 11/01/2007 | 11/14/2009 |
| n/a | 16006 | 1:1,534,076 | 35 | 04/01/2008 | 11/14/2009 |
| US1BS01M | 513 | 1:3,500,000 | 07 | 06/01/2004 | 11/14/2009 |
| n/a | 530 | 1:4,860,700 | 32 | 06/01/2007 | 11/14/2009 |
| n/a | 50 | 1:10,000,000 | 06 | 06/01/2003 | 11/14/2009 |

Table 3 - Charts used during chart comparisons.

No Local Notice to Mariners (LNM) or Notice to Mariners (NTM) affected the survey area.  No features were submitted as Dangers to Navigation (DTON) for the 2009 survey (Appendix I).

The chart comparison was accomplished by generating shoal-biased soundings and contours and overlaying them along with the finalized BASE surfaces on the latest edition NOAA charts.  The general agreement between charted soundings and survey soundings was then examined and a more detailed comparison was undertaken for any shoals or other dangerous features.

General agreement between this survey and the charts is fair.  Significant differences are itemized in the sections below.

    - New Features

The 2009 survey identified no features that are not currently charted within the survey extents.

    - Charted Features

There were no charted features within the survey extents.

    - Soundings

Agreement between charted soundings and surveyed depths is fair, with most depths in general agreement.  Significant discrepancies (generally those greater than +/- 10% of charted depth) are itemized in the table below.  In general, the charted soundings agreed well with the survey.  It is recommended that soundings from Pribilof Shell Hash Survey supersede previously charted soundings, except where noted.

| Chart | Charted Depth | Survey Depth in Vicinity | Charted Position | Comments |
| --- | --- | --- | --- | --- |
| 16381 | 23fm | 27fm | 56-34-58.01N, 169-24-23.92W |  |
| 16381 | 24fm | 21fm | 56-35-06.45N, 169-24-48.39W |  |
| 16381 | 25fm | 21fm | 56-35-33.50N, 169-24-28.28W |  |
| 16381 | 26fm | 21fm | 56-35-48.82N, 169-24-34.09W |  |
| 16381 | 24fm | 21fm | 56-35-40.72N, 169-25-46.12W |  |
| 16381 | 24fm | 21fm | 56-36-18.29N, 169-26-02.69W |  |
| 16381 | 24fm | 21fm | 56-36-31.58N, 169-27-06.45W |  |
| 16381 | 24fm | 21fm | 56-36-50.13N, 169-27-52.52W |  |
| 16381 | 25fm | 21fm | 56-37-46.56N, 169-29-54.25W |  |
| 16382 | 21fm | 25fm | 57-14-50.01N, 169-54-47.79W |  |
| 16382 | 19fm | 16fm | 57-12-00.25N, 170-02-51.82W |  |
| 16011 | 11fm | 16fm | 56-37-29.99N, 169-29-12.11W | Sounding is only partially covered by survey and is not recommended for update. |
| 530 | 16fm | 21fm | 57-06-19.32N, 170-01-22.65W | Sounding is only partially covered by survey and is not recommended for update. |
| 16006 | 2½fm | 18fm | 57-18-24.94N, 169-58-06.97W | Sounding is only partially covered by survey and is not recommended for update. |

Table 4 – 2009 survey soundings which vary significantly from the corresponding charted soundings.

    - Trends and Changeable Areas

Contours were created in Caris Bathydatabase 2.3 and examined concurrently with the charted contours from chart 16381, 16382 and 16380 in ArcMap.

Agreement between contours is fair.  Several of the contours have shifted shoreward.  Several contour lines cross the transit areas, but the hydrographer does not feel there is enough data in these areas to make a recommendation.

Figure 2 – 20fm contours from Pribilof Shell Hash Survey (green) overlaid on chart 16382.

Figure 3 – 40fm contours from Pribilof Shell Hash Survey (green) overlaid on chart 16380.

Figure 4 – 20fm, 30fm and 40fm contours from PRIBILOF SHELL HASH SURVEY (green) overlaid on chart 16381.

The hydrographer recommends that the charted contours be updated to reflect the 2009 survey data, except in the transit areas as noted.

    - AWOIS Items Summary

There were no AWOIS items within the survey extents of the Pribilof Shell Hash Survey.

    - Features Labeled PA, ED, PD, or Rep.

There are no charted features labeled PA, ED, PD or Rep. within the survey extents.

  - Additional Results

    - Aids to Navigation

There were no charted or uncharted Aids to Navigation within the survey extents.

    - Drilling Structures

An investigation of drilling structures is not required under this task order.  No charted or uncharted drilling structures exist within the survey extents.

    - Comparison with Prior Surveys

A comparison with prior surveys was not required under this task order.  See Section D.1 for a comparison to the nautical charts.

    - Bottom Samples

Collection of bottom samples was not required under this task order.

    - Bridges and Overhead Cables

There are no bridges or overhead cables within the survey extents.

    - Submarine Cables and Pipelines

There are no submarine cables or pipelines within the survey extents.

LETTER OF APPROVAL

REGISTRY NUMBER PRIBILOF SHELL HASH SURVEY

This report and the accompanying digital data are respectfully submitted.

Field operations contributing to the accomplishment of the Pribilof Shell Hash survey were conducted under my direct supervision with frequent personal checks of progress and adequacy.  This report, digital data and accompanying records have been closely reviewed and are considered complete and adequate as per the Statement of Work.  Other reports submitted with this survey include the Data Acquisition and Processing Report and the Horizontal and Vertical Control Report.

I believe this survey is complete and adequate for its intended purpose.

_____________________________________

# Chris Popham, Hydrographer

TerraSond Ltd.

01/28/2010

Date_____________________________________