### SIO Sea Beam Merge Format - Documentation

=========================================

[mod 10jun91 smsmith sio/gdc]

Scripps Sea Beam data is  merged  with  navigation  and

stored  on  disk  with  a  record length of 50 16-bit signed

integer words (100 byte).  Comments about the  data  can  be

embedded  in the data, the comments can be recognized by the

year being greater then 7000 (the usual string  to  start  a

comment  with  is  "#").   The comment records are an ascii

string of 100 bytes.  Note that on a earler version  of  the

software  which  ran  on  an IBM 1800, a record of zeros was

used to pad the tape records to 1000 bytes.   These  records

can be recognized by the year being 0 and should be ignored.

SIO merge format data file

logical        data

word           description

---------------------------------------------------------------

time data

1              year (4 digits)

2              julian day (day-of-year, 1-366)

3              no of minutes from beginning of day (0-1439)

4              seconds from beginning of minute (0-59)

position data

5              number of minutes east of prime meridian

6              fraction of minute times 10000

7              number of minutes North of 90 South

8              fraction of minute times 10000

spare

9              unused

10             unused

11             unused

Sea Beam data

12             Sea Beam computer clock time

(known to drift - 1 to 2 minutes per day) It is a

16-bit positive integer representing a count of

10ths of seconds from start of hour

in the range of 0 to 36000. There is no provision

of determining the hour from the data sent by

Sea Beam.

13             heading of ship received from Sea Beam

This is a 16 bit positive integer

000 degrees    = 0

0.0055 degrees = 1

090 degrees    = 16384

180 degrees    = 32768

270 degrees    = 49152

359.99 degrees = 65535

360 degrees    = 0

14-29          16 depths from Sea Beam

In uncorrected meters. using

1500 m/sec. as speed of sound.

30-45          16 cross track distances

In meters from ship, port values

are negative and starboard values

are positive.

position ellipse data (not implemented)

46             azimuth of semi-major axis in degrees

47-48          length of semi-major axis in meters

32bit integer

49-50          length of semi-minor axis in meters

32bit integer

#end doc